Shipping Water Bottles (and other samples) from Remote Locations

Much of PEARL’s work includes research in remote locations, which includes the shipment of water bottles and other types of samples.  Below are some simple suggestions that have been developed over three decades as to how best to get glass bottles back from remote locations using commercial aircraft carriers and other methods of transport.

Typically, especially when we are doing detailed water chemistry, we must return to the lab a large number of different types of water bottles – some glass, some plastic, and in various shapes.

An important thing to check before you leave is to determine what carrier you will use to ship the water bottles back to the lab (e.g. a courier company, an airline, etc.) and find out what are the limits on the weight a package can be. For example, using a very big cooler may seem efficient, but it will be very heavy (and therefore more easily dropped) and may be above the limit for an airline or shipping company.

1) The first important step is the choice of shipping container. For almost all applications, sturdy coolers are preferred. They help maintain temperature of the samples and because they are constructed typically of hard, solid materials, they can take some rough movement (which will inevitably come). Because full coolers will be quite heavy, make sure the coolers are of decent quality (the price of the cooler is insignificant compared to the data that one can lose) and that the handles are sturdy – the cooler will be empty (and light) going into the field, but will be very heavy coming back, full of water.  Good handles (i.e. the best ones appear to those that have a strong handle on two sides of the cooler, not just indentations) also decrease the risk of handlers dropping them.

2) Other things you should have are: 1) a good supply of newspaper for packing, 2) quite a bit of corrugated cardboard (from boxes), 3) duct tape and fibre tape, 4) clear packing tape for labels,  5) permanent markers, 6) labels with addresses and phone numbers for shipping – at least 2 per container, 7) freezer packs to keep samples cool on return journey.

3) At each step of packing, it is important to ask yourself “If this cooler is dropped, what is the best way it can be packed so that nothing breaks”, because invariably it will be dropped, or if you watch baggage handlers move coolers and swing them onto carts etc (regardless of the Fragile stickers), you will see your entire field season at the mercy of others.

4) We ship our empty samples to the Arctic (or whatever) using the same protocols as shipping south (full), and so all our packing material goes to the field and comes back.

5) Once you have your sturdy coolers, we line the entire cooler with corrugated cardboard, as an extra “buffer” against side impacts.

6) Important: Before you put the full water bottle back into the cooler, make sure each lid to each water bottle is tightly on. With field work and movement, lids slowly can become lose. Your last chance to be sure the lid is on as tight as it can be is just before you put it into the cooler for shipment back to the lab.

Also, this is the last time you can check if the bottle is completely and correctly labelled, and the label is securely fastened (a bottle with no label is useless). Certain labs have quite strict protocols on what has to be on the label before they will put the sample into their system. For example, our collaborative work with CCIW requires a special label with specific “schema” written on each label, so that they can flow through the lab system effectively.

7) We often have plastic bottles for some analyses, but glass has to be used for some.  The plastic bottles are best on the outer portions of the cooler, against the walls of the cooler, with glass more inside (protected).  However, it is good to save some plastic bottles to act as “buffers” between some glass bottles, if you have a lot. To some extent, the placing of bottles depends on how many plastic versus glass bottles you have. Ideally, one would always have a plastic bottle in between glass bottles, so that glass bottles are never next to each other.

8) We then wrap each glass bottle in newspaper as extra protection – so typically we double up some newspaper that has a length of about 30-40 cm, and (when doubled up) it is wide enough to protect the whole bottle. We wrap each bottle and place them into cooler.

9) In general, we tend to have groups of no more than 2-3 bottles in a row, before we add more corrugated cardboard as another barrier between the next row of bottles.

10) Once we fill one layer of bottles, we add a floor of more cardboard, and follow the same protocol for the top layer of bottles.

11) If there is any movement in the bottles, fill any spaces with cardboard or newspaper.  The less movement of the bottles, the better.

Remember, if one bottle breaks, you have 2 types of problems. First, other samples will be wet and may lost labels and may also decrease the value of any packing you had (for example, wet newspaper has far less protection than dry newspaper. One option (if feasible) is to carry plastic garbage bags with you into the field, and try and segregate groups of bottles within separate sealed bags in the cooler. If a bottle does break, at least it should not damage too many other bottles.  Needless to say, the more newspaper and cardboard you have in each cooler, that will also absorb any water, if a breakage occurs.

12) Once the cooler is filled, we again put one layer or more of cardboard, and then some frozen freezer packs to try and maintain the temperature as much as possible on the way south.  Fill in any empty spaces with cardboard or newspaper (remembering there is high likelihood the cooler will be tilted on its side or even upside down for some of its travels).

13) The final stages are to make sure the cooler cannot open in its travels and to make sure the labelling is as complete as it can be. Never rely on the latches of the cooler to keep it closed. And remember, no matter how many arrows you might put on a cooler saying “this way up” it will be shipped on its side or any other combination once it is out of your hands.

14) After latching the cooler, we typically “lock” the lid on again by using several (overlapped) turns of fibre tape (much like strapping). If strapping is available, that might be a good idea too.  A ring of duct tape around the top of the cooler acts as a further seal and helps keep it cool.

15) In addition, we routinely label each cooler in 2 places with full name, address, and phone numbers of where the samples are to be delivered. It is also important to have a return address and phone number and contact person, just in case. It is also a good idea to have written here and on other labels: “Glass Water Bottles.  FRAGILE.  Keep cool, but do NOT freeze” (a thick nib red marker is useful for this).  The latter is important for some shipping companies see a cooler and immediately put it in the freezer (frozen glass water bottles will often burst). We secure the labels on the coolers with wide clear packing tape. These labels can be made before you head out to the field and you should make them stand out as much as possible (use colour etc.).

In addition, we add more “Fragile” and “This side up” with arrow labels, in the hope this might help.

Also, it is often a good idea to describe the contents of the coolers so that it can pass through security more easily (the last thing you want is for the coolers to be opened by inspectors, who likely will not take the same care in re-packing the cooler as you did). Simply making clear on a label something like “Contains lakewater samples for scientific research” can be helpful.  If you are shipping sediment samples, writing “Geological samples for scientific research. No commercial value” is what we use.

If you are crossing international borders, much more paperwork will be needed. It is important to do this homework before you go.

It is also useful to number the coolers on the labels (such as “1 of 4 coolers”). This serves 2 purposes.  First, whomever is receiving the coolers will immediately be alerted to a cooler gone astray (if say only 3 of 4 coolers arrive). Second, it is a good idea, if there is time, to keep a record of what is in each cooler.

16) If at all possible, take the full samples with you as checked luggage (and if need be, send your personal luggage as extra) if you are heading back home. You can probably wait a few days for some luggage, but samples (especially in summer heat) are important to get back.

17) If the coolers are being shipped with you on the plane, if at all possible, be at the luggage chute for your flight before the luggage arrives, and wait near the area where the luggage is returned to passengers. In most airports, bags fall down a slide onto a circulating carousel. It is here that many samples can be broken as a full cooler moves with considerable momentum down that slide and bangs into the carousel. If you are there, try and cushion its contact.

18) The above suggestions were primarily for water bottles, but often we also must ship back filters and other samples. Because filters typically have to be kept frozen as much as possible, we often try and take them back with our personal luggage (if we can check it) and wrap the frozen filters in a container (e.g. a zip lock bags) with the freezer pack(s), and then typically wrap the whole thing in a sleeping bag for more insulation.  If you have to ship them in the coolers, we typically have them in the top of the cooler surrounded by freezer packs.

19) We also typically keep the freezer packs in zip lock bags -- one more barrier to possible leakage.

20) Make sure you keep all paperwork, and if samples do not arrive when expected, start investigating. We have 30 years of stories of what happened to samples and what would have happened had we not kept track of them and vigorously pursued their progress during the shipment process!

21) Prior to shipment to the lab doing the analyses, there is typically additional paperwork that must be done for that lab.

22) Also, if we are working in very remote locations, it is not possible to add, for example, concentrated acid to some samples (such as the phosphorus bottles), until we are back south.  As soon as samples have arrived to the lab, add whatever chemicals are needed.

23) Once you ship the samples, be sure to contact the person who will receive the samples with information such as number of coolers, tracking numbers, noting which cooler may have to be opened immediately to remove filters that must go into a freezer, etc.  Avoid shipping perishable samples before a weekend or other period of time when the coolers will simply be sitting in a warehouse.